CMPG 325 milestone 2: client implementation review

AUTHOR: NASIPHI MLAMBO 41555597

Contents

1.Project overview	1

2. Network Design summary	1

2.1  Physical Topology	1

2.2 logical topology(VLANs)	1

2.3 IP Addressing Plan	1

3. Implementation details	2

3.1 Router configuration(R1-Tlhogo)	2

3.2 Switch configuration	2

3.3  Port Security(Assigned Challenge)	2

3.3.1 Reason for Port Security	2

4. Testing evidence	3

4.1 Test 1: PC-Admin – Gateway	3

4.2 Test 2: PC-Admin – pc-projects	3

4.3 Test 3:  PC-Admin-PC-Finance	3

4.4 Test 4: Port Security-Sticky MAC Learned	4

4.5 Test 5: Port Security-Violation Triggered	4

5. Troubleshooting	5

6. Change Request CR5 — 25% Growth	5

# 1.Project overview

Client:  Tlhogo Roofing & Construction (Vryburg)

Client ID: CLI-074

Industry: Construction

Assigned Challenge:	Port Security (switchport access control)

Addressing Block: 192.168.37.0/24

# 2. Network Design summary

## 2.1  Physical Topology

- 1 × Router 2911 (R1-Tlhogo) — Inter-VLAN routing and gateway

- 4 × Switch 2960 (SW-Core, SW-Admin, SW-Projects, SW-Finance)

- 3 × PC (PC-Admin, PC-Projects, PC-Finance)

## 2.2 logical topology(VLANs)

| VLAN ID | Department | Subnet | Gateway | Usable IPs |
| --- | --- | --- | --- | --- |
| 10 | Admin | 192.168.37.0/27 | 192.168.37.1 | 30 |
| 20 | Projects | 192.168.37.32/27 | 192.168.37.33 | 30 |
| 30 | Finance | 192.168.37.64/28 | 192.168.37.65 | 14 |

## 2.3 IP Addressing Plan

| DEVICE | IP ADDRESS | SUBNET MASK | GATEWAY |
| --- | --- | --- | --- |
| PC-Admin | 192.168.37.2 | 255.255.255.224 | 192.168.37.1 |
| PC-Projects | 192.168.37.34 | 255.255.255.224 | 192.168.37.33 |
| PC-Finance | 192.168.37.66 | 255.255.255.240 | 192.168.37.65 |

# 3. Implementation details

## 3.1 Router configuration(R1-Tlhogo)

Sub-interfaces were created for Router-on-a-Stick configuration:

interface GigabitEthernet0/0.10

encapsulation dot1Q 10

ip address 192.168.37.1 255.255.255.224

interface GigabitEthernet0/0.20

encapsulation dot1Q 20

ip address 192.168.37.33 255.255.255.224

interface GigabitEthernet0/0.30

encapsulation dot1Q 30

ip address 192.168.37.65 255.255.255.240

## 3.2 Switch configuration

VLANs created: 10 (Admin), 20 (Projects), 30 (Finance)

Trunk ports configured: SW-Core to router, SW-Core to access switches, SW-Finance to SW-Core

Access ports configured: Each PC assigned to its correct VLAN

## 3.3  Port Security(Assigned Challenge)

Configured on SW-Admin Fa0/2 (the port connecting to PC-Admin):

interface fa0/2

switchport mode access

switchport port-security

switchport port-security maximum 1

switchport port-security violation restrict

switchport port-security mac-address sticky

### 3.3.1 Reason for Port Security

The client constraint states: "Customer records are confidential — access must be controlled."

Port Security prevents unauthorized devices from connecting to the network by locking each switch port to a specific MAC address.

# 4. Testing evidence

## 4.1 Test 1: PC-Admin – Gateway

ping 192.168.37.1

result: 0% loss

## 4.2 Test 2: PC-Admin – pc-projects

ping 192.168.37.34

result: 0% loss

## 4.3 Test 3:  PC-Admin-PC-Finance

ping 192.168.37.66

result: 0% loss

## 4.4 Test 4: Port Security-Sticky MAC Learned

show port-security address

Result: PC-Admin's MAC address locked to Fa0/2

## 4.5 Test 5: Port Security-Violation Triggered

A rogue PC was connected to Fa0/2. Its traffic was blocked:

- show port-security interface fa0/2

result: Security Violation Count : 6

# 5. Troubleshooting

| ISSUE | CAUSE | RESOLUTION |
| --- | --- | --- |
| Packet loss on VLAN 20 | Unused port Fa0/4 flapping in auto mode | Shut down Fa0/4 |
| VLAN 30 unreachable | ARP not resolving | Restarted sub-interface and switches; rebuilt topology |
| STP blocking on trunk | Native VLAN mismatch after port changes | Reconfigured native VLAN 1 on both ends |

# 6. Change Request CR5 — 25% Growth

The addressing plan uses VLSM with the following reserved growth room:

| VLAN | CURRENT NEED | SUBNET SIZE | AVAILABLE | GROWTH ROOM |
| --- | --- | --- | --- | --- |
| 10 | 24 Users | /27 (30 hosts) | 30 | 25% |
| 20 | 24 Users | /27 (30 hosts) | 30 | 25% |
| 30 | 10 Users | /28 (14 hosts) | 14 | 40% |

Additional growth space: 192.168.37.160 – 192.168.37.255 (96 addresses) reserved for future VLANs.